- TAMPERE UNIVERSITY OF TECHNOLOGY

- Institute of Bioelectromagnetism

- Bioelectromagnetism, Exercise 1

- In nerve and muscle cells the concentration ratios of the chloride and potassium ions between intracellular and extracellular fluids are approximately 1:30 (that is, for example ) and 38.8:1. What are the corresponding equilibrium voltages for chloride and potassium ions? (The value of the potential difference across the cell membrane that clamps the specific ionic flow)

- The permeabilities of potassium, sodium and chloride ions in the giant axon of a squid at 36 °C are PK:PNa:PCl = 1:0.04:0.45. The concentrations of the ions in intracellular and extracellular fluids are (10-3 mol/cm3):

[K+]i = 3.45		[K+]o = 0.10
	[Na+]i = 0.72		[Na+]o = 4.55
	[Cl-]i = 0.61		[Cl-]o = 5.40

Determine the resting membrane potential.

- A special single cell organism that lives in a natural mineral water spring has permeabilities of 0.09, 1.00 and 0.04 for the Cl-, K+ and Na+, respectively, and the following concentrations

[Cl-]i = 178		[Cl-]o = 0.47
	[K+]i = 135		[K+]o = 83
	[Na+]i = 0.05		[Na+]o = 118

To maintain the chloride gradient, this cell has a special chloride pump. Which direction        does the pump have to move chloride in order to maintain the status?

- Show that the resting potential of the cell membrane can be expressed as

where

gi = membrane conductance for ions i,

Vi = equilibrium voltage for ions i.

- Figure 1 represents an electrical model of a synapse. When impulse arrives to the presynaptic terminal the transmitter (acetylcholine ACH) is released to the synaptic cleft (after 0.5 ms). The cell membrane will become permeable to Na+ and K+ ions and the membrane potential tends to shift towards the mean of the Nernst voltages of the Na+ and K+ ions (=reversal voltage Vrev).

a) What is the resting potential of the presynaptic terminal (ACH-switch open)?

b) What is the reversal voltage, Vrev (ACH-switch closed)?

Cm (membrane capacitance per area) = 1 µF/cm2
	Nernst voltages VK = -100 mV and VNa = 60 mV
	RK = 108 Ω, RNa = 1.5*109 Ω, DRK = 105 Ω, DRNa = 105 Ω.

Figure 1. Synapse and its' electrical model.