- TAMPERE UNIVERSITY OF TECHNOLOGY

- Institute of Bioelectromagnetism

- Bioelectromagnetism, Exercise 3

- Three electrodes (a, b and c) are on the surface of a volume conductor. Inside the conductor is a dipole source. The lead vectors of this dipole defined at the three locations (a, b and c) are:

The dipole is parallel to the unit vector i. What is the ratio of the voltages measured between electrodes a and b to a and c?

- Derive the lead vectors of the limb leads I, II, and III, leads VR, VF, VL, and the Goldberger leads aVR, aVL ja aVF in a spherical homogeneous volume conductor.

- During a QRS-complex at some time instant t the following potentials were measured:

III-lead		+1,1 mV
		aVR-lead		-3,4 mV
		V1 -lead		-3,5 mV

- Approximate the potentials in the leads I, II, aVL, aVF, V6 and V4.

- On the outer rim of a two dimensional volume conductor on points Pi, i = 1,...,6 the potentials generated by a unit dipole oriented parallel to X or Y axes at point Po are as follows:

Electrode		Potentials generated by X and Y dipoles
	Pi		VX		VY
	1		4		5

2		6		-1
	3		1		-4
	4		-4		-2
	5		-4		2
	6		-1		5

- Using these measurements is it possible to

- Derive the image surface of this source dipole location?

- Calculate the lead vector of a lead between the points 3 and 6?

- Derive the potential at the point 4 generated by a unit dipole at a point Pz?

- Derive the lead field of the volume conductor?

- Construct a VECG lead system (X and Y-leads) that would measure the X and Y components of the dipole at the point Po with similar sensitivity?

- Figure 1 represents a potential data distribution at a model of a (half) head generated by a reciprocal current of 1.0 µA applied to electrodes C and D. Calculate the potential between electrodes C and D generated by current dipoles A and B (|P| = 4 µAcm).